# Project Title

FPGA-Based Traffic Light Controller

# Objective

Design and verify a parameterizable traffic light controller in Verilog with finite-state machine (FSM) timing for a 4-way intersection. Supports vehicle sensors, pedestrian walk request, emergency pre-emption (all-red), night (blinking) mode, debounced inputs, and a clean clock-enable divider for low-power timing. Deliver simulation (Icarus/Verilator), synthesis (Yosys), and an FPGA demo (Artix-7/Cyclone/iCE40).

Prompt

You are an expert VLSI Design Engineer, FPGA Developer, Verilog/System Verilog Engineer, Digital Design Mentor, Verification Engineer, and GitHub Mentor.

Your task is to help me build a complete industry-oriented VLSI project titled:

“FPGA-Based Traffic Light Controller”

IMPORTANT CONTEXT:

- I am a student.

- I want this project to act as proof of work on GitHub.

- This is a VLSI course project.

- I may not have actual FPGA hardware.

- Therefore, provide BOTH:

1. FPGA hardware implementation approach

2. Virtual simulation implementation

I want:

- Complete Verilog/system Verilog code

- Testbench code

- FSM explanation

- Timing diagram

- Simulation steps

- Waveform analysis

- FPGA implementation steps

- GitHub upload strategy

- README

- Screenshots checklist

- Interview preparation

The project must be beginner-friendly but industry-relevant.

--------------------------------------------------

1️⃣ PROJECT EXPLANATION

--------------------------------------------------

Explain:

- What is an FPGA-Based Traffic Light Controller?

- What problem does it solve?

- Why traffic light control is a good FSM-based digital design project?

- How traffic signals can be controlled using digital logic

- How this project demonstrates VLSI and FPGA concepts

Explain:

Simple Explanation

Technical Explanation

Workflow:

Clock Input

↓

Clock Divider

↓

FSM Controller

↓

State Transition Logic

↓

Traffic Light Output

↓

Simulation / FPGA LEDs

--------------------------------------------------

2️⃣ INDUSTRY RELEVANCE

--------------------------------------------------

Explain how similar concepts are used in:

- Traffic control systems

- Embedded controllers

- FPGA-based automation

- Digital control systems

- Industrial automation

- Smart city infrastructure

Explain technical and business value.

--------------------------------------------------

3️⃣ TECH STACK OPTIONS

--------------------------------------------------

Give 3 options:

Option A (Easy)

- Verilog

- Model Sim / EDA Playground simulation

Option B (Recommended)

- Verilog/System Verilog

- Xilinx Vivado

- FPGA board LED implementation

Option C (Advanced)

- FPGA implementation with pedestrian button

- emergency vehicle priority

- sensor-based adaptive timing

For each option explain:

- tools required

- difficulty

- expected output

- whether hardware is required

Then select the best option for students.

--------------------------------------------------

4️⃣ DIGITAL DESIGN CONCEPTS USED

--------------------------------------------------

Explain the role of:

- FPGA

- Verilog/SystemVerilog

- Finite State Machine

- Clock divider

- Sequential logic

- Combinational logic

- Reset signal

- Testbench

- Waveform simulation

Explain purpose of each concept.

--------------------------------------------------

5️⃣ PROJECT ARCHITECTURE

--------------------------------------------------

Design architecture:

Input:

- clock

- reset

- optional pedestrian request

- optional emergency input

Processing:

- clock division

- state transition

- timer counter

- output logic

Output:

- North-South Red/Yellow/Green

- East-West Red/Yellow/Green

- optional pedestrian signal

Include:

- text-based architecture diagram

- FSM state diagram

- state table

- output table

--------------------------------------------------

6️⃣ IMPLEMENTATION PLAN

--------------------------------------------------

Phase 1:

Tool setup

Phase 2:

FSM state planning

Phase 3:

Verilog module design

Phase 4:

Clock divider design

Phase 5:

Traffic light output logic

Phase 6:

Testbench creation

Phase 7:

Simulation and waveform verification

Phase 8:

FPGA constraints file planning

Phase 9:

Synthesis and implementation

Phase 10:

GitHub upload

For each phase explain:

- objective

- tasks

- expected output

- common mistakes

--------------------------------------------------

7️⃣ FOLDER STRUCTURE

--------------------------------------------------

Create:

FPGA-Traffic-Light-Controller/

│

├── rtl/

├── tb/

├── constraints/

├── simulation/

├── waveforms/

├── images/

├── reports/

├── docs/

├── README.md

└── .gitignore

Explain each folder.

--------------------------------------------------

8️⃣ FSM DESIGN

--------------------------------------------------

Design a complete FSM.

Include states such as:

S0: North-South Green, East-West Red

S1: North-South Yellow, East-West Red

S2: North-South Red, East-West Green

S3: North-South Red, East-West Yellow

For each state explain:

- signal output

- duration

- next state

Include:

- FSM diagram in text format

- state transition table

- output encoding

--------------------------------------------------

9️⃣ FULL SOURCE CODE

--------------------------------------------------

Provide complete code:

A. Verilog/SystemVerilog RTL code:

- traffic light controller module

- clock divider if required

- FSM logic

- timer counter

- output signals

B. Testbench code:

- clock generation

- reset testing

- state transition verification

- output monitoring

C. Optional FPGA constraint file example:

- LED pin mapping format

- clock pin example

Code must be:

- modular

- executable

- commented

- beginner-friendly

--------------------------------------------------

🔟 SIMULATION IMPLEMENTATION

--------------------------------------------------

Explain:

- how to simulate in ModelSim

- how to simulate in Vivado

- how to use EDA Playground if software is not installed

- how to run testbench

- how to inspect waveform

- what waveform should show

- how to verify correct state transitions

Mention:

- screenshots to capture

- waveform outputs to save

- simulation logs to upload on GitHub

--------------------------------------------------

1️⃣1️⃣ FPGA IMPLEMENTATION

--------------------------------------------------

Explain:

- how to create Vivado project

- how to add RTL files

- how to add constraints file

- how to run synthesis

- how to run implementation

- how to generate bitstream

- how to program FPGA board

- how LEDs represent traffic lights

If hardware is unavailable:

- explain how simulation acts as proof

--------------------------------------------------

1️⃣2️⃣ REPORT GENERATION

--------------------------------------------------

Generate a project report including:

- project objective

- FSM design

- state table

- RTL explanation

- testbench explanation

- waveform results

- FPGA implementation steps

- conclusion

--------------------------------------------------

1️⃣3️⃣ GITHUB STRATEGY

--------------------------------------------------

Explain:

- repository creation

- commit plan

- screenshots

- waveform images

- documentation

Give:

- best repository name

- best description

- best GitHub tags

--------------------------------------------------

1️⃣4️⃣ README GENERATION

--------------------------------------------------

Generate complete README:

- overview

- problem statement

- VLSI concepts used

- FSM design

- architecture

- tools used

- folder structure

- how to simulate

- sample waveform

- screenshots

- future improvements

- learning outcomes

--------------------------------------------------

1️⃣5️⃣ SCREENSHOTS / PROOF CHECKLIST

--------------------------------------------------

Capture:

- RTL code screenshot

- testbench screenshot

- simulation run screenshot

- waveform screenshot

- FSM diagram screenshot

- state transition table

- synthesis report screenshot

- implementation report screenshot

- FPGA LED output photo/video if hardware is available

- GitHub repository preview

--------------------------------------------------

1️⃣6️⃣ INTERVIEW PREPARATION

--------------------------------------------------

Generate:

- 10 interview questions

- 10 strong answers

IMPORTANT:

One question must be:

“Explain your project.”

FINAL RULES:

- Do not skip RTL code.

- Do not skip testbench.

- Make project simulation-ready.

- Make project GitHub-ready.

- Make project placement-ready.

- Provide copy-paste ready content.

# Industry Relevance

Intersection controllers are classic real-time embedded systems: strict safety sequencing, asynchronous inputs, debouncing, watchdogs, and fail-safe modes. This maps to VLSI/FPGA roles (RTL, CDC, timing closure) and embedded controls (FSM design, verification, diagnostics).

# Step-by-Step Implementation

## Step 1 — Requirements & Top-Level Architecture

What are we doing?
 Specifying behavior and partitioning RTL into timing, IO, and control blocks.

Why are we doing it?
 Clear requirements → traceable FSM, simple verification, easier timing closure.

How do we execute this?

- Directions: North-South (NS) and East-West (EW).

- Signals per leg: G/Y/R LEDs. Pedestrian WALK/DON’T WALK for both crossings.

- Inputs: veh_ns, veh_ew loop sensors (optional), ped_btn, emergency, night_mode, rst_n.

- Timing (defaults, seconds): T_GREEN_MIN=12, T_YELLOW=3, T_ALLRED=1, T_WALK=8, T_FLASH=0.5.

- Modes:

  - Normal: demand-based green (min green then extend by demand).

  - Pedestrian: inserts WALK after the current green phase.

  - Emergency: forces ALL_RED and holds until clear.

  - Night: flashing yellow on main road, flashing red on side road.

- Blocks: clk_div_en (clock enable ticks), debounce_sync, timer, traffic_fsm, outputs_mux.

ChatGPT regeneration prompt
 “List requirements and block diagram for a 4-way traffic controller with pedestrian, emergency, night modes, and parameterized timings.”

## Step 2 — Clocking & Timing Tick (Clock-Enable Divider)

What are we doing?
 Generating a 1 Hz (or 10 Hz) enable tick from the FPGA clock without creating new clock domains.

Why are we doing it?
 Clock-enable style keeps a single clock domain (easier timing/CDC) and reduces toggle/power.

How do we execute this?

// src/clk_en.v

module clk_en #(parameter integer CLK_HZ=50_000_000,

parameter integer TICK_HZ=10)  // 10 Hz tick = 100 ms

(

input  wire clk, rst_n,

output reg  tick

);

localparam integer DIV = CLK_HZ/TICK_HZ;

reg [$clog2(DIV)-1:0] cnt;

always @(posedge clk or negedge rst_n) begin

if(!rst_n) begin cnt<=0; tick<=1'b0; end

else begin

tick <= 1'b0;

if(cnt==DIV-1) begin cnt<=0; tick<=1'b1; end

else cnt<=cnt+1'b1;

end

end

endmodule

ChatGPT regeneration prompt
 “Create a clock-enable divider that outputs a parameterized tick without creating a gated clock.”

## Step 3 — Input Conditioning (Sync + Debounce)

What are we doing?
 Synchronizing and debouncing asynchronous pushbuttons/sensors.

Why are we doing it?
 Avoids metastability and spurious triggers.

How do we execute this?

// src/debounce_sync.v

module debounce_sync #(parameter integer TICKS=5) // TICKS * 100ms with 10Hz tick

(

input  wire clk, rst_n, tick,    // 'tick' from clk_en

input  wire async_in,

output reg  pulse,               // 1-tick pulse on rising edge

output reg  level                // debounced level

);

// 2-FF synchronizer

reg s1,s2; always @(posedge clk) {s1,s2} <= {async_in,s1};

// debounce counter

reg [$clog2(TICKS+1)-1:0] db;

reg stable;

always @(posedge clk or negedge rst_n) begin

if(!rst_n) begin db<=0; stable<=0; level<=0; pulse<=0; end

else begin

pulse<=0;

if(tick) begin

if(s2!=stable) begin db<=db+1; if(db==TICKS) begin stable<=s2; db<=0; end end

else db<=0;

if(stable && !level) begin level<=1; pulse<=1; end

else if(!stable) level<=0;

end

end

end

endmodule

ChatGPT regeneration prompt
 “Write a synchronizer + debouncer that emits a clean pulse and level using the shared tick.”

## Step 4 — Timer & Parameter Pack

What are we doing?
 Creating a general timer counter (in ticks) and centralizing timing parameters.

Why are we doing it?
 Keeps FSM simple; tick-based seconds abstracted.

How do we execute this?

// src/timer.v

module timer(

input  wire clk, rst_n, tick,

input  wire start,

input  wire [15:0] ticks_to_run,

output reg  busy

);

reg [15:0] t;

always @(posedge clk or negedge rst_n) begin

if(!rst_n) begin t<=0; busy<=0; end

else if(start) begin t<=ticks_to_run; busy<=1; end

else if(busy && tick) begin

if(t==0) busy<=0; else t<=t-1;

end

end

endmodule

// src/params.vh

`define T_GREEN_MIN_S   12

`define T_YELLOW_S       3

`define T_ALLRED_S       1

`define T_WALK_S         8

`define T_FLASH_MS     500

ChatGPT regeneration prompt
 “Provide a reusable tick-driven timer and a header for all timing constants in seconds.”

## Step 5 — FSM Specification (States & Transitions)

What are we doing?
 Defining a Moore FSM for normal, pedestrian, emergency, and night sequences.

Why are we doing it?
 A precise graph prevents illegal light combinations and ensures interlocks.

How do we execute this?

- States:
 S_NS_G, S_NS_Y, S_ALL_RED1, S_EW_G, S_EW_Y, S_ALL_RED2, S_PED_WALK, S_PED_CLEAR, S_EMERG_ALL_RED, S_NIGHT_FLASH.

- Transitions (high level):

  - S_NS_G → hold ≥ T_GREEN_MIN; extend if veh_ns present and no ped_req. Then → S_NS_Y.

  - S_NS_Y → T_YELLOW then → S_ALL_RED1.

  - S_ALL_RED1 → S_EW_G.

  - Mirror for EW.

  - If ped_req latched during any green, insert S_PED_WALK (all reds for cars) after the yellow/all-red.

  - If emergency at any time → immediate S_EMERG_ALL_RED until clear (then resume at S_NS_G).

  - If night_mode → S_NIGHT_FLASH (blinking Y on main, blinking R on side).

ChatGPT regeneration prompt
 “List states and narrative transitions for a safe traffic controller including pedestrian, emergency, and night flashing.”

## Step 6 — RTL: Traffic Controller FSM

What are we doing?
 Implementing the parameterized FSM with timers and latches.

Why are we doing it?
 This is the core control logic you’ll simulate and synthesize.

How do we execute this?

// src/traffic_fsm.v

`include "params.vh"

module traffic_fsm #(

parameter MAIN_IS_NS = 1,        // choose main road for night mode

parameter TICK_HZ     = 10

)(

input  wire clk, rst_n, tick,

input  wire veh_ns, veh_ew, ped_pulse, emergency, night_mode,

output reg  ns_g, ns_y, ns_r,

output reg  ew_g, ew_y, ew_r,

output reg  ped_walk, ped_dontwalk

);

typedef enum logic [3:0] {

S_NS_G=0, S_NS_Y, S_ALL_RED1, S_EW_G, S_EW_Y, S_ALL_RED2,

S_PED_WALK, S_PED_CLEAR, S_EMERG_ALL_RED, S_NIGHT_FLASH

} state_t;

state_t s, s_n;

// Latches & timers

reg ped_req_latched;

reg [15:0] wait_ticks; reg start_t; wire busy_t;

timer u_t(.clk(clk),.rst_n(rst_n),.tick(tick),.start(start_t),.ticks_to_run(wait_ticks),.busy(busy_t));

// time helpers (convert seconds->ticks)

function [15:0] SEC(input integer s); SEC = s*(TICK_HZ); endfunction;

function [15:0] MS (input integer ms); MS  = (ms*TICK_HZ)/1000; endfunction;

// Pedestrian request latch on pulse

always @(posedge clk or negedge rst_n)

if(!rst_n) ped_req_latched <= 1'b0;

else if(ped_pulse) ped_req_latched <= 1'b1;

else if(s==S_PED_CLEAR) ped_req_latched <= 1'b0;

// State register

always @(posedge clk or negedge rst_n) begin

if(!rst_n) s <= S_NS_G;

else if(emergency) s <= S_EMERG_ALL_RED;

else if(night_mode) s <= S_NIGHT_FLASH;

else s <= s_n;

end

// Next-state and timer control

always @* begin

// defaults

s_n = s; start_t = 1'b0; wait_ticks = 16'd0;

// outputs default (set below)

{ns_g,ns_y,ns_r, ew_g,ew_y,ew_r, ped_walk,ped_dontwalk} = 8'b0;

unique case (s)

S_NS_G: begin

{ns_g, ew_r, ped_dontwalk} = 3'b111;

if(!busy_t) begin start_t=1'b1; wait_ticks=SEC(`T_GREEN_MIN_S); end

else if(busy_t) ; // still within min green

else ;            // placeholder

// after min green, allow exit on (no veh_ns OR ped_req OR veh_ew)

if(!busy_t && (ped_req_latched || veh_ew)) s_n = S_NS_Y;

end

S_NS_Y: begin

{ns_y, ew_r, ped_dontwalk} = 3'b111;

if(!busy_t) begin start_t=1'b1; wait_ticks=SEC(`T_YELLOW_S); end

else if(!busy_t) s_n = S_ALL_RED1;

if(!busy_t && wait_ticks==0) s_n = S_ALL_RED1;

end

S_ALL_RED1: begin

{ns_r, ew_r, ped_dontwalk} = 3'b111;

if(!busy_t) begin start_t=1'b1; wait_ticks=SEC(`T_ALLRED_S); end

else if(!busy_t) s_n = ped_req_latched ? S_PED_WALK : S_EW_G;

end

S_EW_G: begin

{ew_g, ns_r, ped_dontwalk} = 3'b111;

if(!busy_t) begin start_t=1'b1; wait_ticks=SEC(`T_GREEN_MIN_S); end

if(!busy_t && (ped_req_latched || veh_ns)) s_n = S_EW_Y;

end

S_EW_Y: begin

{ew_y, ns_r, ped_dontwalk} = 3'b111;

if(!busy_t) begin start_t=1'b1; wait_ticks=SEC(`T_YELLOW_S); end

if(!busy_t && wait_ticks==0) s_n = S_ALL_RED2;

end

S_ALL_RED2: begin

{ns_r, ew_r, ped_dontwalk} = 3'b111;

if(!busy_t) begin start_t=1'b1; wait_ticks=SEC(`T_ALLRED_S); end

if(!busy_t) s_n = ped_req_latched ? S_PED_WALK : S_NS_G;

end

S_PED_WALK: begin

{ns_r, ew_r, ped_walk} = 3'b111;

if(!busy_t) begin start_t=1'b1; wait_ticks=SEC(`T_WALK_S); end

if(!busy_t && wait_ticks==0) s_n = S_PED_CLEAR;

end

S_PED_CLEAR: begin

{ns_r, ew_r, ped_dontwalk} = 3'b111;

if(!busy_t) begin start_t=1'b1; wait_ticks=SEC(`T_ALLRED_S); end

if(!busy_t) s_n = S_NS_G; // resume cycle

end

S_EMERG_ALL_RED: begin

{ns_r, ew_r, ped_dontwalk} = 3'b111;

// stay until emergency deasserts; state reg handles preemption release -> S_NS_G

end

S_NIGHT_FLASH: begin

// blink main road Y, side road R using 0.5s toggles

ped_dontwalk = 1'b1;

if(MAIN_IS_NS) begin

ns_y = tick; ns_r = ~tick; // simple blink pattern

ew_r = tick; ew_y = ~tick;

end else begin

ew_y = tick; ew_r = ~tick;

ns_r = tick; ns_y = ~tick;

end

// exit when night_mode deasserts (handled in state reg)

end

default: s_n = S_NS_G;

endcase

end

endmodule

ChatGPT regeneration prompt
 “Implement a Moore FSM for traffic control with timers, pedestrian insertion, emergency all-red, and night flashing.”

## Step 7 — Top-Level Integration & Pinout

What are we doing?
 Wiring tick, debouncers, FSM, and IO pins for an FPGA board.

Why are we doing it?
 Gives a working bitstream demo on common dev boards.

How do we execute this?

// src/top.v

module top(

input  wire clk_50m, rst_btn,

input  wire veh_ns_raw, veh_ew_raw, ped_btn_raw, emergency_raw, night_raw,

output wire NS_G, NS_Y, NS_R, EW_G, EW_Y, EW_R,

output wire PED_WALK, PED_DONT

);

wire rst_n = ~rst_btn;

// 10 Hz tick

wire tick;

clk_en #(.CLK_HZ(50_000_000), .TICK_HZ(10)) u_tick(.clk(clk_50m), .rst_n(rst_n), .tick(tick));

// Debounce/sync inputs

wire ped_pulse, ped_level, em_pulse, em_level, night_level, vns_level, vew_level;

debounce_sync #(.TICKS(3)) u_ped(.clk(clk_50m),.rst_n(rst_n),.tick(tick),.async_in(ped_btn_raw),.pulse(ped_pulse),.level(ped_level));

debounce_sync #(.TICKS(1)) u_em (.clk(clk_50m),.rst_n(rst_n),.tick(tick),.async_in(emergency_raw),.pulse(em_pulse),.level(em_level));

debounce_sync #(.TICKS(1)) u_ng (.clk(clk_50m),.rst_n(rst_n),.tick(tick),.async_in(night_raw),   .pulse(),.level(night_level));

debounce_sync #(.TICKS(1)) u_vns(.clk(clk_50m),.rst_n(rst_n),.tick(tick),.async_in(veh_ns_raw), .pulse(),.level(vns_level));

debounce_sync #(.TICKS(1)) u_vew(.clk(clk_50m),.rst_n(rst_n),.tick(tick),.async_in(veh_ew_raw), .pulse(),.level(vew_level));

// FSM

traffic_fsm #(.MAIN_IS_NS(1), .TICK_HZ(10)) u_fsm(

.clk(clk_50m), .rst_n(rst_n), .tick(tick),

.veh_ns(vns_level), .veh_ew(vew_level), .ped_pulse(ped_pulse),

.emergency(em_level), .night_mode(night_level),

.ns_g(NS_G), .ns_y(NS_Y), .ns_r(NS_R),

.ew_g(EW_G), .ew_y(EW_Y), .ew_r(EW_R),

.ped_walk(PED_WALK), .ped_dontwalk(PED_DONT)

);

endmodule

ChatGPT regeneration prompt
 “Integrate tick, input conditioning, and the FSM in a top module targeting a 50 MHz FPGA clock.”

## Step 8 — Testbench & Simulation (Self-Checks + Waveforms)

What are we doing?
 Creating a behavioral TB with accelerated timing to validate sequences and safety (never green both ways).

Why are we doing it?
 Catches logic issues before synthesis; produces waveforms for reports.

How do we execute this?

// tb/traffic_tb.v

`timescale 1ns/1ps

module traffic_tb;

reg clk=0, rst_btn=1;

reg veh_ns=0, veh_ew=0, ped=0, emer=0, night=0;

wire NS_G,NS_Y,NS_R,EW_G,EW_Y,EW_R,PW,PD;

// Use smaller "clock" and overspeed tick for simulation

top DUT(.clk_50m(clk), .rst_btn(rst_btn), .veh_ns_raw(veh_ns), .veh_ew_raw(veh_ew),

.ped_btn_raw(ped), .emergency_raw(emer), .night_raw(night),

.NS_G(NS_G),.NS_Y(NS_Y),.NS_R(NS_R),.EW_G(EW_G),.EW_Y(EW_Y),.EW_R(EW_R),

.PED_WALK(PW), .PED_DONT(PD));

always #5 clk = ~clk; // 100 MHz sim clock

// sanity: never both greens

always @(posedge clk) begin

if(NS_G && EW_G) begin

$display("ERROR: conflicting greens!"); $finish;

end

end

initial begin

$dumpfile("traffic.vcd"); $dumpvars(0,traffic_tb);

#100 rst_btn=0;

// simulate demand: EW vehicle and pedestrian press

#20000 veh_ew=1; #10000 veh_ew=0;

#15000 ped=1; #20000 ped=0;

// emergency pulse

#20000 emer=1; #20000 emer=0;

// night mode

#40000 night=1; #40000 night=0;

#200000 $finish;

end

endmodule

ChatGPT regeneration prompt
 “Write a testbench that asserts ‘never both directions green’, drives veh/ped/emergency/night, and dumps a VCD.”

## Step 9 — Synthesis & FPGA Constraints

What are we doing?
 Synthesizing with Yosys; adding pin constraints for a popular board (Artix-7).

Why are we doing it?
 To produce a bitstream and map LEDs/buttons to real IO.

How do we execute this?

Yosys (generic)

 # scripts/synth.ys

read_verilog src/clk_en.v src/debounce_sync.v src/timer.v src/traffic_fsm.v src/top.v

hierarchy -top top

synth_xilinx -flatten -top top

stat

write_json build/traffic.json

XDC (example, Nexys A7)

 set_property PACKAGE_PIN W5  [get_ports clk_50m]   ; # 50MHz

set_property IOSTANDARD LVCMOS33 [get_ports clk_50m]

set_property PACKAGE_PIN U18 [get_ports rst_btn]

# LEDs (map as needed)

set_property PACKAGE_PIN U16 [get_ports NS_G]

set_property PACKAGE_PIN E19 [get_ports NS_Y]

set_property PACKAGE_PIN U19 [get_ports NS_R]

set_property PACKAGE_PIN V19 [get_ports EW_G]

set_property PACKAGE_PIN W18 [get_ports EW_Y]

set_property PACKAGE_PIN U15 [get_ports EW_R]

set_property PACKAGE_PIN V14 [get_ports PED_WALK]

set_property PACKAGE_PIN V13 [get_ports PED_DONT]

# Buttons/switches for inputs...

- For iCE40 (UP5K) use synth_ice40 and .pcf instead.

ChatGPT regeneration prompt
 “Provide a Yosys script and example XDC/PCF hints to map LEDs/buttons on a real FPGA board.”

## Step 10 — Safety, Diagnostics & Extensions

What are we doing?
 Hardening and proposing meaningful upgrades.

Why are we doing it?
 Real intersections demand fail-safe behavior and serviceability.

How do we execute this?

- Safety

  - Output interlock: assert that (NS_G|NS_Y) implies EW_R, and vice-versa.

  - Watchdog: if FSM stuck in a state > 2× expected time → force S_EMERG_ALL_RED.

  - Power-up default: all red for ≥ T_ALLRED.

- Diagnostics

  - 7-segment/UART status: show state ID and remaining seconds.

  - Event log counters: pedestrian served, emergency count, watchdog trips.

- Extensions

  - Adaptive green: extend with measured queue length sensor.

  - Coordination: sync multiple controllers over UART/SPI for green waves.

  - RTC-based schedule: switch to night mode by time-of-day.

  - AXI-lite wrapper: control from a soft-core CPU (MicroBlaze/RISC-V).

  - Formal assertions (SymbiYosys): prove no conflicting greens.

ChatGPT regeneration prompt
 “List safety interlocks, a watchdog plan, and five extensions (adaptive green, networking, RTC, CPU control, formal proofs).”

# Project Facts

Market Growth 📈
 Smart traffic systems and ITS electronics continue growing with urbanization and EV adoption; FPGAs are favored for deterministic control + rapid iteration.

Cost Savings 💰
 FPGA prototyping reduces time-to-field testing; adaptive timing improves throughput and lowers idling fuel costs in real deployments.

Equipment Longevity ⚙️
 Clock-enable (single domain) and debounced inputs reduce spurious toggles and stress on IO/power rails.

Real-World Applications 🌍
 Campus/park intersections, factory floors, parking facilities, and teaching labs.

Industry Adoption 🏭
 Municipal ITS vendors and automotive testbeds prototype intersection logic on FPGAs before ASIC/PLC deployment.

Interview Question and Answer

-- 1. Interview Question: Explain your FPGA-Based Traffic Light Controller project.

-- Answer:

-- In this project, I designed a traffic light controller using Verilog and implemented it as a finite state machine. The system controls North-South and East-West traffic signals by switching between green, yellow, and red states based on predefined timing. I verified the design using a testbench and waveform simulation, and the same logic can be implemented on an FPGA board using LEDs to represent traffic lights.

-- 2. Interview Question: Which VLSI concepts are used in this project?

-- Answer:

-- This project uses finite state machine design, sequential logic, combinational logic, clock division, counters, reset logic, Verilog RTL coding, testbench creation, and waveform-based verification.

-- 3. Interview Question: Why did you use an FSM for traffic light control?

-- Answer:

-- A traffic light controller works through a fixed sequence of states. FSM is suitable because each traffic condition can be represented as a state, and the controller moves from one state to another based on timing logic.

-- 4. Interview Question: What are the main states in your design?

-- Answer:

-- The main states are North-South Green with East-West Red, North-South Yellow with East-West Red, North-South Red with East-West Green, and North-South Red with East-West Yellow.

-- 5. Interview Question: What is the role of the clock in this project?

-- Answer:

-- The clock controls the timing of state transitions. Each state remains active for a specific number of clock cycles before moving to the next state.

-- 6. Interview Question: Why is a clock divider used?

-- Answer:

-- FPGA boards usually have high-frequency clocks. A clock divider is used to slow down the clock so that traffic light changes can be observed clearly on LEDs.

-- 7. Interview Question: What is the role of reset?

-- Answer:

-- Reset initializes the controller to a known starting state. In this project, reset brings the traffic light controller back to the initial state, such as North-South Green and East-West Red.

-- 8. Interview Question: How did you verify your design?

-- Answer:

-- I verified the design using a Verilog testbench. The testbench generates clock and reset signals and observes whether the traffic light outputs change correctly according to the FSM sequence.

-- 9. Interview Question: What output does your project generate?

-- Answer:

-- The project generates traffic light outputs for North-South and East-West directions, including red, yellow, and green signals. In simulation, these outputs are verified through waveforms, and on FPGA they can be mapped to LEDs.

-- 10. Interview Question: How can this project be improved further?

-- Answer:

-- This project can be improved by adding pedestrian crossing signals, emergency vehicle priority, sensor-based adaptive timing, countdown display, multiple road junction support, and real-time traffic density-based control.